Environmental Science and Pollution Research

https://doi.org/10.1007/s11356-023-28451-7

RESEARCH ARTICLE

Fabrication of ordered layered SnO2/

TiO2 heterostructures and their

photocatalytic performance for methyl blue degradation

Walid Serbout1 · Fayçal Bensouici2 · Omar Meglali1,3 · Sabrina Iaiche2 · Mohamed Bououdina4 · Steffano Bellucci5 ·

Muhammad Humayun4

Received: 7 April 2023 / Accepted: 22 June 2023

© The Author(s), under exclusive licence to Springer-Verlag GmbH Germany, part of Springer Nature 2023

Abstract

The rapid growth in population and industrialization has given rise to serious environmental issues, especially the water pollution. Photocatalysis with the assist of semiconductor photocatalysts has been considered as an advanced oxidation technique for degrading a variety of pollutants under solar irradiation. In this work, we have fabricated SnO2- TiO2 heterostructures with different ordered layers of SnO2 and TiO2 via the sol–gel dip-coating technique and utilized in photocatalysis for degradation

of methyl blue dye under UV irradiation. The influence of the layer’s position on SnO2 and TiO2

properties is investigated via the various techniques. The grazing incidence X-ray diffraction (GIXRD) analysis reveals that the as-prepared films exhibit pure anatase TiO2 and kesterite SnO2 phases. The 2SnO2/2TiO2 heterostructure exhibit the maximum crystallite sizeand smallest deviation from the ideal structure. Scanning electron microscopy cross-section images manifest good adhesionof the layers to each other and to the substrate. Fourier transform infrared spectroscopy reveals the characteristic vibration

modes of SnO2 and TiO2 phases. UV–visible spectroscopy measurements indicate that all films exhibit high transparency (T = 80%) and the SnO2 film reveals a direct band gap of 3.6 eV, while the TiO2 film exhibits an indirect band gap of 2.9 eV. The optimal 2SnO2/2TiO2 heterostructure film revealed best photocatalytic degradation performance and the reaction rateconstant for methylene blue solution under UV irradiation. This work will trigger the development of highly efficient heterostructure photocatalysts for environmental remediation.

Keywords Dip-coating ・ Heterostructure ・ Ordered layers ・ Photocatalysis ・ Methyl blue

Introduction

Global dye production exceeds 7 × 105 t, which is used not only in the textile sector but also in the food, paper, and pharmaceutical industries (Srivastava et al. 2017). It is estimated that about 15% of these dyes are lost during the synthesis, dye processing, dyeing, printing, and finishing processes (Aye et al. 2003). Being common industrial pollutants, the decolorization and mineralization of the dyes

via different techniques become necessary. The decomposition of dye molecules may result in the formation of dye fragments, that might be more toxic and environmentally persistent compared to the actual dyes such as aromatic amines. It was reported that conventional physical, chemical,

and biological treatment methods for decolorization and decomposition are ineffective, because of their low degradation efficiency, chemical consumption, and high operation costs (Abdel-Messih et al. 2013; Mehrdad and Hashemzadeh 2010). To minimize these limitations,

Responsible Editor: George Z. Kyzas

* Muhammad Humayun

mhumayun@psu.edu.sa

1 Materials Science and Informatics Laboratory, Ziane Achour

University, 17000 Djelfa, Algeria

2 Laboratory of Structures, Properties and Intermolecular

Interactions LASPI2A, Department of Matter, Abbes

Laghrour University, 40.000 Khenchela, Algeria

3 Faculty of Sciences, Mohammed Boudiaf University,

M’Sila 28000, Algeria

4 Energy, Water, and Environment Lab, College of Humanities

and Sciences, Prince Sultan University, Riyadh 11586,

Saudi Arabia

5 INFN-Laboratori Nazionali Di Frascati, Vie. E. Fermi 54,

00044 Frascati, Italy